ZCLOG DAQ

Wireless Data Acquisition Software

Version: 3.1

Users Manual

ZHICE-ELEC

BY USING THIS SOFTWARE PRODUCT IN ANY MANNER, YOU ARE AGREEING TO ACCEPT THEFOLLOWING TERMS AND CONDITIONS.

HEFEI ZHICE ELECTRONIC CO.,LTD (ZHICE-ELEC) grants you a non-exclusive right to use ZCLOG VALIDATION Software (Product) on a single PC or workstation (node), or on multiple nodes, provided that a multi-use license is obtained from ZHICE-ELEC and the number of nodes does not exceed the number of nodes for which license fees are paid. This grant of license does not include the right to copy, modify, rent, lease, sell, transfer or distribute the Product or any portion thereof. You may not reverse engineer, decompile, or disassemble the Product.

ZHICE-ELEC warrants that the Product will perform in its intended environment substantially in accordance with the accompanying written materials for a period of 90 days from the date of license acceptance. ZHICE-ELEC does not warrant any downloading errors or that the Product will be error free or operate without interruption.

ZHICE-ELEC DISCLAIMS ALL OTHER WARRANTIES, EITHER EXPRESS OR IMPLIED. BUT NOT LIMITED TO IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. WITH RESPECT TO THE SOFTWARE AND THE ACCOMPANYING WRITTEN MATERIALS. In no event shall ZHICE-ELEC be liable for any damages whatsoever (including, without limitation, indirect, consequential, or incidental damages, damages for loss of business profits, business interruption, loss of business information, or other pecuniary loss) arising out of the use

Users Tips

- The resolution of the device running this software should not be lower than 1280x800. For excellent interface interaction, it is recommended to adjust the computer's display resolution to above 1280*800 and the display ratio setting to be 100%.

- This software requires ZCLOG IF-02 model adapter and USB to COM adapter communication driver auxiliary software support. Please install these necessary auxiliary software as required before starting and running this software.

- Before use, it is recommended to go through this manual in detail and follow the relevant instructions. During use, click [Help] for timely user support.

- In the manual, anything circled with ‘’ represents a menu, everything circled with [] represents a button, and → represents a continuous action.

- Please note that the abbreviation "ZCLOG DAQ" is used instead of the software in this article.

Table of Content

Chapter 1 Installation & Uninstallation	1

1.1 Software Installation	1

1.1.1 ZCLOG IF-01/02Adapter communication interface driver software installation	1

1.1.2 “ZCLOG DAQ” Program installation	6

Chapter 2 Function Module Introduction	13

2.1 System settings	13

2.1.1 Basic Settings	13

2.2  Data Collection	14

2.2.1 Logger Status	14

2.2.2 Test Parameter	16

2.2.3 Collection Settings	18

2.2.4 Data Downloads	21

2.2.5 Data Report	22

2.3 About	26

Contact Us	26

# Chapter 1 Installation & Uninstallation

## 1.1 Software Installation

Reminder: The software only supports Win7-64-bit, Win10, and Win11 operating systems.

The software installation package will contain the following files:

Adapter driver

Software User manual

Software installer

Sunclient remote control tool

### ZCLOG IF-01/02Adapter communication interface driver installation

This driver file is shown in the figure below.

To install the two files "ftdibus.inf" and "ftdiport.inf",

Step1：Use the matching USB communication cable to connect the USB port of the computer and the port of the adapter. When using it for the first time, the driver has not yet been installed. The "FT232R USB UART" logo as shown below will be displayed in the “Device Manager” of the computer. If your Computer Management - Device Manager - Port location automatically refreshes the new port number, which means that your computer has previously installed a similar driver, you can skip the subsequent steps and use it directly.

Step2：In the device manager interface, right-click to select "FT232R USB UART" and select "Update Driver Software" to find the driver folder. First install "ftdibus.inf":

After the dialog box is closed successfully, the Device Manager interface will refresh, as shown in the following picture.

Step3: After the installation is successful, repeat step2, update the driver, and install “ftdiport.inf”. After the installation is successful, the Device Manager can correctly identify the adapter model as shown in the figure below.

Additional note: The driver contains two parts, the device manager interface corresponding to the "Universal Serial Bus Controller" and "Ports". The corresponding driver files of these two hardware parts are respectively the "ftdibus.inf" and "ftdiport.inf" two files in the driver folder. So only when both two parts are successfully installed indicates that the adapter driver is fully installed. Users may encounter installation failure when installing the driver for the first time, you can contact us for help in time.

### “ZCLOG DAQ” Program installation

Find the "ZCLOG_DAQ_V3.0_Setup_x64.exe" program, double-click it and complete the installation according to the following picture.

The file directory can be modified, or you can use the default, and then click 'Next'.

You are advised to select the “Create Desktop Shortcut” option and then click “Next”.

Select “Install” to start installation.

The following dialog box will pop up and click "Finish" to complete the program installation of the software.

At this point, the software installation step has been completed. Before you start your official use of the software, the following suggestions are recommended to follow:

- Revise the user permission of the software installation directory

Right-click the software icon "ZCLOG DAQ" on the desktop, open the file location, and find the software installation directory folder "ZCLOG DAQ"; Right-click this folder - Properties - Security - Edit - Select "Users" account in "Group or User Name" - Check "Full Control" in the right box below and clieck “OK”.

- English version usage notice

Users from non-Mainland China, when using the English version of this software, please note that your computer system parameters must be consistent as shown below, otherwise your software interface may appear garbled!

# Chapter 2 Function Module Introduction

Note: The software is authorized to be used through registration, and the following window will pop up when opening the software for the first time to remind the user to register. According to the prompts in the window, the user can log in to use the software after successful registration.

## 2.1 System settings

### 2.1.1 Basic Settings

Enter 'System Settings' and 'Basic Settings', enter relevant information in the box, and click the [OK] button. The basic information will be saved in the built database file, and will be displayed in the data report later.

Data Length: Used to set the number of decimal places in the data.

Note: The language change needs to  restarted the software to take effect.

## 2.2  Data Collection

### 2.2.1 Logger Status

Before setting the logger, please configure the port and adapter type in this interface first, and then perform other setting operations if the query is normal, to ensure that the logger status and communication are normal.

Port Number: The drop-down box will display the serial port recognized by the current computer. Please check your computer's device manager to determine the port number used by your adapter.

Note: The port number cannot exceed 16!

Adapter type: M-Adatper-10, used for ZC02 serials logger!  10 loggers can be placed in a M-Adatper-10. Please be careful not to insert the backwards. Then click the query button, the successful connection will display the information of the current recorder, if the query fails, please check the adapter connection.

Note: It is recommended to query before setting each project.

Check whether the logger information is normal, such as battery status. The fully charge logger status is generally displayed between 3.5~3.8V. If the battery power is lower than 2.8V, it will affect the logger during high or low temperature data collection. Please contact our company's after-sales personnel for battery replacement.

Coll-Status: we have 3 kind of status, waiting,stop,collect.

- 2.2.2 Collection Settings

Note: Before setting collections, check the communication and connection of the adapter and logger to prevent the interruption of the setting process due to abnormal communication.

Number:  set the number you need to set in this collection project.

Interval:  set the seconds of collect interval.

Duration:  Set the collection total duration use hour & minutes.

Timing:  Check to use the function. You can set a delay time. When the set time is reached, the logger automatically begins to collect data. The delay time must exceed the current computer display time by at least 5 minutes. If "Timing " is not selected, the logger will start collecting data immediately after the setting is complete.

Click "Next" and the logger setting information will pop up, as shown in the picture above. After all the Settings are successful, click the "Finish" button.

### 2.2.3 Data Downloads

After the setting of Step 2.2.3 is successful, the created test project will be displayed on the left side of the data download interface in the following figure, indicating that the project data has not been completed and has not been downloaded to the database by the software. Select the item in the list box on the left and the logger information under the item will be displayed in the list box on the right. Click the "Download" button on the right to save the data to the database for subsequent report functions. After all logger data for an item has been successfully downloaded, the area on the left no longer displays the item information. Project data needs to be viewed on the report interface. The invalid item on the left can also be deleted from the report interface. Once the item is deleted, the download interface will no longer display the corresponding item information.

### 2.2.4  Data Report

The query box in the upper left corner supports keyword query. If you do not enter any information, then the software query all the items in the database. Select the item on the left and right-click or double-click to view it. You can view the data of a single or multiple loggers under the same project at a time. You cannot view more than two projects at the same time.

Report module related functions:

Trend chart:

The left vertical axis of the trend chart is fixed to the temperature value, and the right vertical axis is displayed according to your recorder type. When your project has THS humidity data but no TPS pressure data, the right side will be displayed in terms of humidity. When your data includes the pressure data of the TPS, the right vertical axis will be displayed according to the pressure regardless of whether the humidity data is also included.

The trend chart supports scaling: Hold down the left mouse button in any position in the trend chart and drag the mouse to select the right box to enlarge the graph in the selected area. If the box is selected in the left direction, the original scale is retracted.

Channels can be selected in the trend chart: If you deselect a probe channel, the data of the channel is not displayed in the trend chart and will not be displayed in the exported report. Please note: Unchecking this location does not affect the display of the right axis ordinate name. For example, the illustrated item contains both TPS and THS recorders. In the initial state, the name of the right axis is "pressure kPa". If all TPS recorders are unchecked here, the name of the right axis will not be refreshed as "Humidity %RH". If you want the right axis to become humidity, you need to uncheck the TPS recorder in the left item list, and double-click or right-click to re-view the data, and then the name of the right axis will be displayed as "humidity %RH".

Display Range Setting area: the horizontal and vertical coordinates of the icon can be set inside this function box. If there is a TPS or THS recorder, the temperature ordinate will be displayed on the left, and the corresponding pressure or humidity coordinate will be displayed on the right; TPS and THS loggers do not exist in the same project at the same time, and when THS loggers are used, the bar setting for the value axis becomes the %RH setting.

Export Excel：Export the original data of the EXCEL file, the file content is simple, only contains the project name information and recorder data, convenient for users to the original data for other functions of the secondary processing.

You can generate reports through the export button or print button in the upper left corner of the preview window in the figure below. The export button supports exporting reports in three formats: PDF. The print button supports virtual machines or local paper printers.

## 2.3 About

Clicking the "Help Document" button will bring up the instruction manual for the software (this feature requires that you have Microsoft Office installed properly on your computer). You can also view the help documents in the software installation package or installation directory. During the installation and use of the software, if you encounter any problems, please contact our company.

- Contact Us

Hefei Zhice Electronics Co., Ltd.

Service: QQ：3241802098

Whatsapp: +86 18788840124

Web:www.ZHICE-ELEC.com

Tel：+86 4001890661

Email: info@zhice-elec.com

Add:D-7, No.168 XIANGZHANG ROAD, HEFEI,ANHUI,CHINA